COMMENCING AT THE CENTERLINE INTERSECTION OF MARTIN L. KING BOULEVARD AND LAKE MEAD BOULEVARD; THENCE…

(The terms COMMENCING, BEGINNING, POINT OF BEGINNING and POINT OF TERMINATION should always be bold)

EXPLANATION:

THIS LAND DESCRIPTION DESCRIBES A PARCEL OF LAND GENERALLY LOCATED AT THE SOUTHWEST CORNER OF LAS VEGAS BOULEVARD AND CLARK AVENUE.

MAP REFERENCES:

THAT SUBDIVISION KNOWN AS “CLARK’S LAS VEGAS TOWNSITE” ON FILE IN THE OFFICE OF THE COUNTY RECORDER, IN BOOK 1 OF PLATS, AT PAGE 37

THAT RECORD-OF-SURVEY ON FILE IN THE OFFICE OF THE COUNTY RECORDER, IN FILE 193 OF SURVEYS, AT PAGE 94

THE ABOVE DESCRIBED PARCEL OF THE LAND IS DEPICTED ON THAT RECORD OF SURVEY, ON FILE IN THE OFFICE OF THE COUNTY RECORDER, IN FILE 78 OF SURVEYS, AT PAGE 80

DOCUMENT/DEED REFERENCES:

(DEED / DEDICATION / DOCUMENT or other possibilities) RECORDED… MAY 24, 2011, IN THE OFFICE OF THE COUNTY RECORDER AS INSTRUMENT NO. 20110524:04749 IN OFFICIAL RECORDS

THE (THAT CERTAIN) LAND DESCRIBED BY THAT DEED RECORDED…

(Pre-1954)

RECORDED DECEMBER 28, 1948, IN THE OFFICE OF THE COUNTY RECORDER IN BOOK 59 OF DEEDS, AT PAGES 97 THROUGH 98, AS INSTRUMENT NO. 354076

(Old Form)

RECORDED JANUARY 17, 1974, IN THE OFFICE OF THE COUNTY RECORDER IN BOOK 395 OF OFFICIAL RECORDS, AS INSTRUMENT NO. 354076

(Current Form)

RECORDED AUGUST 29, 2007, IN THE OFFICE OF THE COUNTY RECORDER, AS INSTRUMENT NO. 20070829:02294

CURVES:

TO THE BEGINNING OF A TANGENT CURVE, CONCAVE WESTERLY AND HAVING A RADIUS OF 10.00 FEET; THENCE NORTHWESTERLY ALONG SAID CURVE, 15.70 FEET THROUGH A CENTRAL ANGLE OF 89°58’54”

TO THE BEGINNING OF A NON-TANGENT CURVE, CONCAVE WESTERLY AND HAVING A RADIUS OF 10.00 FEET, A RADIAL LINE TO SAID BEGINNING BEARS NORTH 31º53’35” WEST; THENCE NORTHWESTERLY ALONG SAID CURVE, 15.70 FEET THROUGH A CENTRAL ANGLE OF 89°58’54”

TO A POINT OF CUSP WITH A CURVE CONCAVE WESTERLY AND HAVING A RADIUS OF 10.00 FEET, A RADIAL LINE TO SAID POINT OF CUSP BEARS NORTH…

(All radial line bearings should be expressed from the center out)

DOUBLE-CALLS:

THENCE SOUTH 27º55’16” WEST, ALONG THE EAST LINE OF SAID LOT 5, 741.16 FEET TO A POINT ON THE WEST LINE OF SAID LOT 5

THENCE NORTH 62º07’30” WEST, DEPARTING SAID EAST LINE AND ALONG SAID WEST LINE, 354.79 FEET;

TO THE BEGINNING OF A TANGENT CURVE, CONCAVE WESTERLY AND HAVING A RADIUS OF 10.00 FEET; THENCE NORTHWESTERLY ALONG THE EAST LINE OF SAID LOT 5 AND ALONG SAID CURVE, 15.70 FEET THROUGH A CENTRAL ANGLE OF 89°58’54

POINT REFERENCES:

TO THE NORTHEAST CORNER OF SAID LOT 5, SAME BEING THE POINT OF BEGINNING.

TO THE NORTHEAST CORNER OF SAID LOT 5, ALSO BEING ON THE SOUTH RIGHT-OF-WAY LINE OF MAIN STREET (80 FEET WIDE)

(SAME and ALSO are interchangeable)

STRIP DESCRIPTIONS:

BEING A STRIP OF LAND, 20.00 FEET WIDE, LYING 10.00 FEET ON BOTH SIDES OF THE FOLLOWING DESCRIBED CENTERLINE:

(Statement should be in its own paragraph placed before the point of commencement)

THENCE NORTH 88°24’14” EAST 310.05 FEET TO THE WESTERLY RIGHT-OF-WAY LINE OF C STREET, ALSO BEING THE POINT OF TERMINATION.

THE SIDE LINES OF SAID STRIP TO BE SHORTENED OR LENGHTED TO BEGIN AT THE SAID WEST LINE OF LOT 2, TO MEET AT ANGLE POINTS AND TO TERMINATE AT THE SAID EAST LINE OF LOT 2

(The following language may be considered unnecessary, but may be required by certain entities)

THE SIDE LINES OF SAID STRIP TO BE SHORTENED OR LENGHTED TO BEGIN AT A LINE PERPINDICULAR TO THE POINT OF BEGINNING, TO AND TERMINATE AT A LINE PERPINDICULAR TO THE POINT OF ENDING.

RIGHT-OF-WAY ADJACENT TO ANNEXATION:

TOGETHER WITH THE SURROUNDING HALF STREETS OF FIRST AVENUE AND SECOND ROA

EXCEPTIONS:

EXCEPTING THEREFROM ALL AREAS FALLING BELOW AN ELEVATION OF 2042 FEET, BEING A POINT A MINIMUM OF 16 FEET ABOVE THE HIGHEST CROWN ELEVATION OF THE “BONNEVILLE CLARK ONEWAY COUPLET PHASE 1” IMPROVEMENT PLANS, BETWEEN BONNEVILLE AVENUE AND CLARK AVENUE, AS SHOWN ON CITY OF LAS VEGAS DRAWING NUMBER 107V5201.

FURTHER EXCEPTING THEREFROM THAT PARCEL CONVEYED TO THE CITY OF LAS VEGAS, A MUNICIPAL CORPORATION FOR RIGHT-OF-WAY PURPOSES BY THAT DOCUMENT RECORDED AUGUST 29, 2007, IN THE OFFICE OF THE COUNTY RECORDER, AS INSTRUMENT NO. 20070829:02294.

VACATION RESERVATIONS:

RESERVING THEREFROM AN EASEMENT TO NEVADA POWER COMPANY D/B/A NV ENERGY OVER, UNDER, THROUGH AND ACROSS THE PUBLIC RIGHT OF WAY BEING VACATED WITHIN THE LIMITS OF THE VACATION AS DECRIBED HEREIN.

ALSO RESERVING AND EXCEPTING THEREFROM AN EASEMENT TO SOUTHWEST GAS CORPORATION ON, OVER, IN, UNDER, ACROSS, ABOVE AND THROUGH WITHIN THE LIMITS OF THE VACATION AS DECRIBED HEREIN.

ALSO RESERVING THEREFROM AN EASEMENT TO THE LAS VEGAS VALLEY WATER DISTRICT (DISTRICT) OVER, UNDER, THROUGH AND ACROSS THE AREA BEING VACATED, WITHIN THE LIMITS OF THIS VACATION AS DECRIBED HEREIN.

STAND-ALONE LOT & BLOCK DESCRIPTION:

BEING PARCEL 2 AS SHOWN ON THAT PARCEL MAP ON FILE IN THE OFFICE OF THE COUNTY RECORDER, IN FILE 131 OF PARCEL MAPS, AT PAGE 38, LYING WITHIN THE NORTHEAST QUARTER (NE 1/4) OF THE NORTHEAST QUARTER (NE 1/4) OF SECTION 36, TOWNSHIP 20 SOUTH, RANGE 61 EAST. M.D.M., CITY OF LAS VEGAS, CLARK COUNTY, NEVADA.

CONTAINING 3.01 ACRES, AS SHOWN ON SAID MAP.

BASIS OF BEARINGS STATEMENT (MAP REFERENCE):

BASIS OF BEARINGS:

SOUTH 00°15’12” EAST, BEING THE BEARING OF THE EAST LINE OF THE NORTHEAST QUARTER (NE 1/4) OF THE NORTHWEST QUARTER (NW 1/4) OF SECTION 21, TOWNSHIP 20 SOUTH, RANGE 61 EAST, M.D.M., CITY OF LAS VEGAS, CLARK COUNTY, NEVADA, ALSO BEING THE CENTERLINE OF MARTIN LUTHER KING BOULEVARD, AS SHOWN ON THAT RECORD OF SURVEY ON FILE IN THE OFFICE OF THE COUNTY RECORDER, IN FILE 148 OF SURVEYS, AT PAGE 79.